INTRODUCTION

This document describes the various switches that control the functionality of the Director’s Chair (DC) and Director’s Tool (DT) and to a lesser extent the actual Window’s driver itself.  These features are functional beginning with Windows driver version 1.6.0.

The DT and DC use switch settings from two files:

SYSTEM.INI

PROMTN.INI

The SYSTEM.INI is created when the user installs the Windows environment and the PROMTN.INI is created when the user installs the ProMotion Windows 3.x driver.  Let’s discuss the SYSTEM.INI settings first.

SYSTEM.INI description

In the SYSTEM.INI a paragraph heading called [ProMotion] is created if not present by the ProMotion setup program.  In this section you will find the entries that the DC, DT and the driver create.  The information in this section should not be altered as it may cause unusual behavior.  The entries are as follows:

[ProMotion]

Display= 		name of display monitor selected

Mode_640x480= 	current refresh rate for this resolution

Mode_800x600=	current refresh rate for this resolution

Mode_1024x768=	current refresh rate for this resolution

Mode_1152x864=	current refresh rate for this resolution

Mode_1280x1024=	current refresh rate for this resolution

Mode_1600x1200=	current refresh rate for this resolution

Resolution=		current resolution

ResOnFly=		enable resolution on the fly

CursorColor1=	RGB values for color cursor

Director’s Tools=	coordinates of the Director’s Tools

DisableAutoLoad =	force auto-loading of director’s chair / director’s tool

The setup program will create a line in the WIN.INI file called ‘load = PROMCHAR.EXE’.  You must remove this line, if present, to properly disable the auto loading.[v1.04.00+]

PROMTN.INI description

The PROMTN.INI file is copied from the Windows’ setup disk to the user’s PC; it will overwrite an existing PROMTN.INI file if present on the user’s PC.  If this file is not present on the setup disk during run-time the setup program will terminate with an error.  This file is distributed to the OEM with the optimal settings for the ProMotion chip.  The file contains configuration information for: FIFO, maximum refresh rates and available resolutions.   So the paragraph headings one would see if they opened the PROMTN.INI is the following:

[FIFO Overrides]

[Max Refresh Rates]

[Resolution Preferences]

[CRTC Registers]

FIFO Overrides

The Windows 3.x driver is configured to support FIFO settings when MCLK is 50.0 MHz and 60.0MHz.  If the OEM sees the need to adjust these settings for a given user or users, they can override the driver FIFO settings by placing the appropriate values in this section.  In the current the PROMTN.INI the override values are commented out, because they will have no effect since the Windows driver contains the exact same values.  Use extreme caution when modifying these values, as Alliance assumes no responsibility what-so-ever if OEMs change these values with out first consulting with Alliance.

Max Refresh Rates

Here the OEM can dictate to the Windows’ driver what the maximum refresh should be for 8 [xx], 15/16 [yy] and 32 [zz] bit color depth.  This will override the user selection if the refresh rate supplied here is lower that the one selected by the user.  The following entries are recognized:

mode_640x480= 	[xx,][yy,][zz]

mode_800x600=	[xx,][yy,][zz]

mode_1024x768=	[xx,][yy,][zz]

mode_1152x864=	[xx,][yy,][zz]

mode_1280x1024=	[xx,][yy,][zz]

mode_1600x1200=	[xx,][yy,][zz]

The brackets around the variables indicate that that this value is an option, however to skip a color depth, a comma must be supplied.  For example to set the refresh rates for 640x480 at 8bpp to 72 Hz and 32bpp to 75 Hz the entry would look like this:

mode_640x480=72,,75

Resolution Preferences

This section allows the OEM to control which resolutions the user can configure his setup to.  This section is left blank by Alliance so that all resolutions are available to the user.  The following entries are recognized:

Res_640x480=	a 0,1 or no entry for this screen resolution

Res_800x600=	a 0,1 or no entry for this screen resolution

Res_1024x768=	a 0,1 or no entry for this screen resolution

Res_1152x864=	a 0,1 or no entry for this screen resolution

Res_1280x1024=	a 0,1 or no entry for this screen resolution

Res_1600x1200=	a 0,1 or no entry for this screen resolution

A one [1] or a no entry for a particular screen resolution indicates that the resolution will not be disabled by the software.  If there is a value of ‘0’ [zero] for a particular screen resolution that will cause the software to remove the selection from the user’s list in the Director’s Chair, Director’s Tool and the ProMotion driver setup program.

CRTC_Registers

This section contains the appropriate CRTC values to center the display.  Entries only appear if they have been changed by the user.  Color/Bit depth is no relevant.

640x480x60= 	value for Horizontal Retrace Start(3D5.4), value for Horizontal Retrace End (3D5.5), value 				for Vertical Overflow (3D5.7), value for Retrace Start (3D5.10), value for Vertical Retrace End 				(3D5.11) or no entry for this screen resolution and refresh rate.

640x480x72= 	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 					refresh rate.

640x480x75= 	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

800x600x56=	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

800x600x60=	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

800x600x72= 	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

800x600x75= 	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1024x768x60=	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1024x768x70=	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1024x768x72= 	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1024x768x75= 	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1152x864x66=	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1152x864x72= 	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1152x864x75=	value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1280x1024x60= value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1280x1024x72= value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1280x1024x75= value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1600x1200x60= value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

1600x1200x70= value for 3D5.4, 3D5.5, 3D5.7, 3D5.10, and 3D5.11 or no entry for this screen resolution and 				refresh rate.

A blank or a missing entry for a particular screen resolution indicates that the user did not change the settings for  that resolution .